Loading

#### SPECIFICATIONS

Cities in the U.S. generally base their building

code on one of the three model codes:

- Uniform Building Code

- Basic Building Code (BOCA)

- Standard Building Code

# Loading

These codes have been consolidated in the 2000 International Building Code.

Loadings in these codes are mainly based on ASCE Minimum Design Loads for Buildings and Other Structures (ASCE 7-95) – has been updated to ASCE 7-98.

# Dead Loading

- Weight of all permanent construction

- Constant magnitude and fixed location

Examples:

# Dead Load

  - Weight of the Structure

(Walls, Floors, Roofs, Ceilings, Stairways)

  - Fixed Service Equipment

(HVAC, Piping Weights, Cable Tray,	Etc.)

##### Can Be Uncertain…

  - pavement thickness

  - earth fill over underground structure

# Live Load

    - Loads produced by use and occupancy of the structure.

    - Maximum loads likely to be produced by the intended use.

    - Not less than the minimum uniformly distributed load given by Code.

# Live Load

See Table 2-1 from ASCE 7-95

Stairs and exitways:	100 psf Storage warehouses:	125 psf (light)

250 psf (heavy)

Minimum concentrated loads are also given in the codes.

# Live Loads

Live Load

ASCE 7-95 allows reduced live loads for members with influence area (AI) of 400 sq. ft. or more:

L    L

o





 0 . 25





A

I





where Lo

 0.50 Lo for members



15	

supporting one floor

 0.40 Lo otherwise

# Live Loads

AI determined by raising member to be designed by a unit amount. Portion of loaded area that is raised = AI

Beam:	AI = 2 * tributary area

Column:	AI = 4 * tributary area Two-Way Slab:	AI = panel area

(see Fig. 2-10, text)

# Load Reduction

Environmental  Load

- Snow Loads

- Earthquake

- Wind

- Soil Pressure

- Ponding of Rainwater

- Temperature Differentials

### Classification of Buildings for Wind, Snow and Earthquake Loads

#### Based on Use Categories (I through IV)

- Buildings and other structures that represent a low hazard to human life in the event of a failure (such as agricultural facilities)

- Buildings/structures not in categories I, III, and IV

### Classification of Buildings for Wind, Snow and Earthquake Loads

#### Based on Use Categories (I through IV)

- Buildings/structures that represent a substantial hazard to human life in the event of a failure (assembly halls, schools, colleges, jails, buildings containing toxic/explosive substances)

### Classification of Buildings for Wind, Snow and Earthquake Loads

#### Based on Use Categories (I through IV)

- Buildings/structures designated essential facilities (hospitals, fire and police stations, communication

centers, power-generating stations)

# Snow Load

The coefficients of snow loads are defined in weight.

# Snow Load

Ground Snow Loads (Map in Fig. 6, ASCE 7):

  - Based on historical data (not always the maximum values)

  - Basic equation in codes is for flat roof snow loads

  - Additional equations for drifting effects, sloped roofs, etc.

  - Use ACI live load factor

  - No LL reduction factor allowed

# Wind Load

    - Wind pressure is proportional to velocity squared (v2 )

    - Wind velocity pressure = qz

qz  

0.00256 Kz

kzt

V 2 I

# Wind Load

2

where

qz  

0.00256 Kz

kzt V	I

0.00256 reflects mass density of air and unit conversions.

V =	Basic 3-second gust wind speed (mph) at a height of 33 ft. above the ground in open terrain. (1:50 chance of exceedance in 1 year)

Kz =	Exposure coefficient (bldg. ht., roughness of terrain) kzt =	Coefficient accounting for wind speed up over hills I	=	Importance factor

# Wind Load

Design wind pressure,

p = qz * G * Cp

| G = | Gust Response Factor |  |
| --- | --- | --- |
| Cp = | External pressure coefficients (accounts for directions on building) | pressure |

# Earthquake Load

Inertia forces caused by earthquake motion

F = m * a

- Distribution of forces can be found using equivalent

static force procedure (code, not allowed for every

building) or using dynamic analysis procedures

# Earthquake Load

Inertia forces caused by earthquake motion.	Equivalent Static Force Procedure for example, in ASCE 7-95:

where

V = Cs * W

V =	Total lateral base shear

Cs =	Seismic response coefficient W =	Total dead load

# Earthquake Load

Total Dead Load, W:

1.0 * Dead Load

+ 0.25 * Storage Loads

+ larger of partition loads or 10 psf

+ Weight of permanent equipment

+ contents of vessels

+ 20% or more of snow load

## Earthquake Loads

1.2 C

2.5 C

C   smaller of

	v

and 	a

s	 R T

2 / 3	R

where

Cv =	Seismic coefficient based on soil profile and Av Ca =	Seismic coefficient based on soil profiled and Aa

R =	Response modification factor (ability to deform in inelastic range)

T =	Fundamental period of the structure

## Earthquake Loads

1.2 C

2.5 C

C   smaller of

	v

and 	a

s

where

 R T

2 / 3	R

T =	Fundamental period of the structure

T = CT hn 3/4

where CT =	0.030 for MRF of concrete

0.020 for other concrete buildings. hn =	Building height

# Earthquake Map

Roof Load

- Ponding of rainwater

  - Roof must be able to support all rainwater that could accumulate in an area if primary drains were blocked.

  - Ponding Failure:

 Rain water ponds in area of maximum deflection

 increases deflection

 allows more accumulation of water  cycle continues… potential failure

# Roof Load

    - Roof loads are in addition to snow loads

    - Minimum loads for workers and construction materials during erection and repair

# Construction Loads

    - Construction materials

    - Weight of formwork supporting weight of fresh concrete