Lab 2

Second stage

Medical Physical Department

Digital Electronics

Lab2: Basic logic Gates (AND, OR, and NOT gates)

By

Asst. Prof. Dr. Mehdi Ebady Manaa

# Basic logic Gates (AND, OR, and NOT gates)

## Objectives

1- To study and understand the 3 basic gates. 2- Implement the basic gate in EWB.

3- The study the specifications of every gate when connected it with one input constant and the other is variable.

### AND and NAND gates

This gate gives high output (1) if all the inputs are 1’s. otherwise the output will be low (0).

Its Boolean algebra representation is: C=A.B

And it’s truth table and schema as following:

| A | B | C |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

The NAND gate works opposite to the AND gate. Its Boolean algebra representation is: C=(A.B)' And it’s truth table and schema as following:

| A | B | C |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

### OR and NOR gates

This circuit will give high output (1) if any input is high (1).

Its Boolean algebra representation is:	C=A+B

and it’s truth table and schema as following:

| A | B | C |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

The NOR gate works opposite to the OR gate. Its Boolean algebra representation is: C=(A+B)'  And it’s truth table and schema as following:

| A | B | C |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 0 |

### NOT gate

This is the simplest gate it just inverts the input, if the input is high the output will be low and conversely.

So B=A’

| A | B |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

## Lab Tasks

### Task 1: The AND and NAND gates

In EWB, draw the following two circuits and fill the truth table below

| A | B | A.B | (A.B)' |
| --- | --- | --- | --- |
| 0 | 0 |  |  |
| 0 | 1 |  |  |
| 1 | 0 |  |  |
| 1 | 1 |  |  |

### Task 2: The AND-NOT combination

In EWB, draw the following circuit and fill the truth table

| A | B | (A.B)’ |
| --- | --- | --- |
| 0 | 0 |  |
| 0 | 1 |  |
| 1 | 0 |  |
| 1 | 1 |  |

### Task 3: The OR and NOR gates

In EWB, draw the following two circuits and fill the truth table below

| A | B | A+B | (A+B)’ |
| --- | --- | --- | --- |
| 0 | 0 |  |  |
| 0 | 1 |  |  |
| 1 | 0 |  |  |
| 1 | 1 |  |  |

### Task 4: The NOR-NOT combination

| A | B | ((A+B)’)’ |
| --- | --- | --- |
| 0 | 0 |  |
| 0 | 1 |  |
| 1 | 0 |  |
| 1 | 1 |  |

### Task 5: Finding the truth table of a three input gate using the logic converter

Repeat what you did in task 5 for a three-input AND gate. Show your connections in the circuit below. Note: you can obtain a three-input AND gate by drawing a regular two-input AND gate and then changing its Number of Inputs property as shown next.

| A | B | C | D |
| --- | --- | --- | --- |
| 0 | 0 | 0 |  |
| 0 | 0 | 1 |  |
| 0 | 1 | 0 |  |
| 0 | 1 | 1 |  |
| 1 | 0 | 0 |  |
| 1 | 0 | 1 |  |
| 1 | 1 | 0 |  |
| 1 | 1 | 1 |  |