كلية العلوم

قسم الفيزياء الطبية

MEDICAL PHSICS 
المرحلة : الثانية

المادة : البصريات

اسم التجربة:  " Law s' Snell "

رقم التجربة : 1

مشرف المختبر: د. دعاء جعفر ضياء

معيدي المختبر : م.م حمزة                                                              م.حسن هادي

:The purpose of the experiment

- Study the refraction of light as it travels between two different media.

Verify Snell's law (law of refraction) -

Introduction :-

: Theory (Detailed Explanation)

: Refraction Phenomenon

When light travels from one transparent medium to another (e.g., air to glass), its

speed changes due to the difference in optical density, causing the light to bend at

the interface between the two media. This bending of light is called refraction.

: Snell's Law

Snell's Law mathematically describes the relationship between the angles of

: incidence and refraction and the refractive indices of the two media

n1 sin(θ1) = n2 sin(θ2)

:Where

- n1: Refractive index of the first medium (air, typically n1 = 1).

- n2: Refractive index of the second medium (glass).

- θ1: Angle of incidence, measured between the incident ray and the normal to the

surface.

- θ2: Angle of refraction, measured between the refracted ray and the normal.

:Tools used

1.  Light source (laser)

2.  Transparent or rectangular semicircle made of glass or plastic.

3.  Protractor (for measuring angles).

4.  Graph paper.

5.  Pencil and ruler.

:Refractive Index

:The refractive index n is defined as the ratio of the speed of light in a vacuum (c) to

its speed in a given medium (v)

n = c/v

The greater the refractive index, the slower the light travels in the medium and the

greater the bending of light at the interface.

T

Practical Application:

By measuring the angles of incidence (θ1) and refraction (θ2) for light passing

through the glass, the refractive index of the glass can be calculated using:

n2 = sin(θ1) / sin(θ2)

Factors Affecting Refraction:

1. Wavelength of Light: Red light bends less than blue light due to its longer

wavelength.

2. Temperature: Changes in temperature can alter the density of glass and its

refractive index.

3. Material Purity: Impurities or irregularities in the glass can slightly affect its

refractive index.

-

Procedure:

1. Setup: - Place the glass piece on a sheet of white paper on a flat surface. - Stabilize the light source to produce a precise beam.

2. Measure the Angle of Incidence: - Direct the light beam onto the glass surface at a specific angle (θ1) measured using the protractor.

. - Mark the point of incidence and draw the incident ray’s path on the paper.

3. Measure the Angle of Refraction: - Observe the refracted beam inside the glass and mark its exit point.

- Draw the refracted ray’s path and measure the angle of refraction (θ2) using the protractor.

4. Record the results in the table as show below

5.Calculate the Refractive Index:

- Repeat the experiment for different angles of incidence