قــســــــــــم الأمــــــــــــــــــــــــــن الــــــــــــــــــــــــــــــــــــسيبرانــــــــــــــــــــــــــــــــــــي

Department of Cyber Security

Subject:

MICROPROCESSORS

Lecture: (2)

microprocessor

Class: Second

Lecturer: M.Sc.Muntather AL-mussawee

Microprocessor Pin

Address/Data bus

Low order address bus

these are multiplexed with data.

When AD lines are used to transmit memory address the symbol A is used Instead at AD,

for example A0-A15

When data are transmitted over AD lines the symbol D ls used in place to AD,

for example  D0-D7  D8-D15  or  D0-D15

AD16/S3                                AD18/S5

AD17/S4                                AD19/S6

High order address bus

These are multiplexed with status signals

34) BHE/S7(Active Low)/S,(Output)

Bus High Enable/Status

It is used to enable data onto the most significant half of data bus,

D8-D15, 8-blt device connected to upper half of the data bus use (BHE)

(Active Low) signal. It is multiplexed with status signal S,.

33) MN/ MX

MINIMUN/ MAXIMUN

This pin signal indicates what mode the processor is to operate in.

32) RD (Read) (Active Low)

The signal is used for read operation.

It is an output signal

It is active when low.

23) TEST

TEST Input is tested by the (WAIT)   instruction.

8086 will enter a wait state after execution

of the WAIT instruction and will resume execution only when the TEST is made low by an active hardware.

This is used to synchropters an external

activity to the Processor internal operation.

22) READY

It is available at pin 22. It is an acknowledgement signal from I/O devices that data is transferred. It is an active high signal. When it is high, it indicates that the device is ready to transfer data. When it is low, it indicates wait state

21) RESET (Input)

Causes the processor to immediately terminate its present activity.

The signal must be active HIGH for at least four clock cycles.

19) CLK

Clock signal is provided through Pin-19. It provides timing to the processor for operations. Its frequency is different for different versions, i.e. 5MHz, 8MHz and 10MHz

18) INTR Interrupt Request

It is available at pin 18. It is an interrupt request signal, which is sampled during the last clock cycle of each instruction to determine if the processor considered this as an interrupt or not

Min/ Max Pins

The 8068 microprocessor can work in two modes of operations: Minimum mode and Maximum mode

In the minimum mode of operation the Microprocessor do not associate with any co-processors and can not be used for

multiprocessor systems

In the maximum mode the 8086 can work in multi-processor or

Co-processor configuration.

Minimum or maximum mode operations are decided by the

pin MN/ MX(Active low).

When this pin is high 8086 operates in minimum mode

otherwise it operates in Maximum mode.

Minimum mode signals

Pins 24-31

For minimum mode operation , the MN/ MX is tied

to VCC ( logic high)

8086 itself generates all the bus control signals

27) DT/ R̅ (Data Transmit/ Receive)

It stands for Data Transmit/Receive signal and is available at pin 27. It decides the direction of data flow through the trans receiver. When it is high, data is transmitted out and vice-a-versa

26) DNE

(Data Enable) Output signal from the processer

used as output enable for the transceivers.

25) ALE

(Address Latch Enable) Used to de multiplex the

address and data lines using external latches.

28) M/ IO

Used to differentiate memory access and I/O access. for memory reference instructions, It is high. For IN and OUT instructions , It is Low

29) WR

Write control signal; asserted low Whenever processor writes data to memory or I/0 port.

24) INTA

(interrupt Acknowledge) When the interrupt request is accepted by the processor , the output is low on this line.

31) HOLD

Input signal to the processor form the bus masters as a request to grant the control of the bus.

Usually used by the DMA controller to get the control of the bus.

30) HLDA

(hold Acknowledge) Acknowledge signal by the processor to the bus master requesting the central of the bas through HOLD.

The acknowledge is asserted High, when the

processor accepts HOLD.

Maximum mode signals

During maximum mode operation, the MN/ MX is grounded (logic low)

Pins 24--31 are reassigned

S0 , S1 , S2

Status signals ; used by the 8086 bus controller to

generate bus timing and control signals. There are

decoded as shown :

QS1, QS0

(Queue Status) The processer provides the status

of queue in these lines.

The queue status can be used by external device to

track the internal status of the queue in 8066.

The output on QS0  and QS1 can be interpreted as shown in the table

RQ / GT1

RQ / GT0

These are the Request/Grant signals used by the other processors requesting the CPU to release the system bus. When the signal is received by CPU, then it sends acknowledgment. RQ/GT0 has a higher priority than RQ/GT1.