Department of  Electrical  Engineering techniques

Ministry of Higher Education and

Scientific Research – Iraq

AL-Mustaqbal University

DC Generators

المحاضرة 9و8

. Characteristics of D.C. Generators

اعداد

الدكتور جابر غايب  القاضي

# Introduction

The speed of a d.c. machine operated as a generator is fixed by the prime mover. For general-purpose operation, the prime mover is equipped with a speed governor so that the speed of the generator is practically constant. Under such conditions, the generator performance deals primarily with the relation between excitation, terminal voltage and load. These relations can be best exhibited graphically by means of curves known as generator characteristics. These characteristics show briefly the behavior of the generator under different load conditions.

يختلف مولد التيار المباشر عن مولد التيار المتردد بالعديد من الخصائص التي تشمل التركيب الدقيق، وذلك بالإضافة الى آلية الفحص التي يتعرض لها، وذلك على خلاف ما جرت العادة في مولدات القدرة الكهربائية التي تعمل بشكل مصاحب لشبكة النقل الكهربائية طوال الوقت، لذلك سوف يتم في هذا الطرح توضيح أهم الخصائص الكهربائية وإجراءات الفحص المناسبة لهذا النوع من المولدات.

بالإضافة الى أن مولد التيار المباشر (DC)

هو عبارة عن “آلة دوارة” تزود ناتجاً كهربائياً بجهد وتيار أحادي الاتجاه، حيث أن المبادئ الأساسية للتشغيل هي نفسها للمولدات المتزامنة، كما يتم تحفيز الجهد في الملفات بمعدل تغير المجال المغناطيسي عبر الملفات أثناء دوران الآلة. هذا الجهد المستحث يتناوب بطبيعته في الشكل، وذلك لأن تدفق الملف يزداد ثم يتناقص، وبمتوسط قيمة صفرية.

ومن أجل ضمان متطلبات التصميم؛ فإنه عادةً ما يتم صناعة مولدات التيار المستمر من أربعة إلى ثمانية أقطاب مجال، وجزئياً لتقصير “الوصلات الطرفية” على ملفات الدوار وجزئياً لتقليل كمية الحديد المغناطيسي اللازمة في الجزء الثابت، وعادة ما يكون عدد الفرش الثابتة التي تحمل على المبدل الدوار مساوياً لعدد الأعمدة ولكن قد يكون اثنان فقط في بعض التصميمات.

Characteristics of D.C. Generators

The d.c. generator have following characteristics in general:

(1)No-Load (or Magnetization) Characteristic.

(2)Load Characteristic, which is subdivided into;

(i) Internal Characteristic

(ii) External Characteristic

Separately-excited Generator

- No-load Saturation Characteristic (E0/If)

  - Load Saturation Curve (V/If)

The curve showing relation between the terminal voltage V and field current If when the generator is loaded, is known as Load Saturation Curve.

The curve can be deduced from the no-load saturation curve provided the values of armature reaction and armature resistance are known. While considering this curve, account is taken of the demagnetizing effect of armature reaction and the voltage drop in armature which are practically absent under no-load conditions. Under load conditions, the voltage will decrease due to demagnetizing effect of armature reaction. This decrease can be made up by suitably increasing the field amp-turns.

  - Internal and External Characteristics

Considering a separately excited generator giving its rated no-load voltage of E0 for a certain constant field current. If there were no armature reaction and armature voltage drop, then this voltage would have remained constant as shown in Fig. 2 by the horizontal line I. But when the generator is loaded, the voltage falls due to these two causes, thereby giving slightly dropping characteristics.

If we subtract from E0 the value of voltage drops due to armature reaction for different loads, then we get the value of E–the e.m.f. actually induced in the armature under load conditions. Curve II is plotted in this way and is known as the internal characteristic. The straight line Oa represents the IaRa drops corresponding to different armature currents. If we subtract from E the armature drop IaRa, we get terminal voltage V. Curve III represents the external characteristic and is obtained by subtracting ordinates the line Oa from those of curve II.

# Characteristics of a Shunt Generator

Fig (4) (i) shows the connections of a shunt wound generator. The armature current Ia splits up into two parts; a small fraction Ish flowing through shunt field winding while the major part IL goes to the external load.

  - Voltage Build-Up

Before loading a shunt generator, it is allowed to build up its voltage. Usually, there is always present some residual magnetism in the poles, hence a small e.m.f. is produced initially. This e.m.f. circulates a small current in the field circuit which increases the pole flux (provided field circuit is properly connected to armature, otherwise this current may wipe off the residual magnetism). When flux is increased, generated e.m.f. is increased which further increases the flux and so on. As shown in Fig. 3, Oa is the induced e.m.f. due to residual magnetism which appears across

the field circuit and causes a field current Ob to flow. These current aids residual flux and hence produces, a larger induced e.m.f. Oc. In turn, this increased e.m.f. Oc causes an even larger current Od which creates more flux for a still larger e.m.f. and so on.

Now, the generated e.m.f. in the armature has

(a) to supply the ohmic drop ifRf in the winding and (b) to overcome the opposing self-induced e.m.f. in the field coil i.e. Ldifdt because field coils have appreciable self-inductance.

| Eg | = | ifRf+ Ldifdt | (3) |
| --- | --- | --- | --- |

Fig. 3

If (and so long as), the generated e.m.f. is more than the ohmic drop ifRf, energy would continue being stored in the pole fields. For example, as shown in Fig. 3, corresponding to field current OA, the generated e.m.f. is AC. Out of this, AB goes to supply ohmic drop If Rsh and BC goes to overcome self-induced e.m.f. in the coil. Corresponding to If = OF, whole of the generated e.m.f. is used to overcome the ohmic drop. None is left to overcome Ldifdt. Hence no energy is stored in the pole fields. Consequently, there is no further increase in pole flux and the generated e.m.f. With the given shunt field resistance represented by line OP, the maximum voltage to which the machine will build up is OE. If resistance is decreased, it will build up to a somewhat higher voltage. OR represents the resistance known as critical resistance. If shunt field resistance is greater than this value, the generator will fail to excite.

  - O.C.C.

The O.C.C. of a shunt generator is similar in shape to that of a series generator as shown in Fig. (4) (ii). The line OA represents the shunt field circuit resistance. When the generator is run at normal speed, it will build up a voltage OM. At no-load, the terminal voltage of the generator will be constant (= OM) represented by the horizontal dotted line MC.

اعزائي الطلبة ارسل لكم بعض الشروحات بالعربي للمساعدة بفهم المحاضرة

خصائص مولد التيار المستمر المستحث بشكل منفصل

Fig. 4

  - Internal characteristic

When the generator is loaded, flux per pole is reduced due to armature reaction. Therefore, e.m.f. E generated on load is less than the e.m.f. generated at no load. As a result, the internal characteristic (E/Ia) drops down slightly as shown in Fig.4 (ii).

  - External characteristic

Curve 2 shows the external characteristic of a shunt generator. It gives the relation between terminal voltage V and load current IL.

| V | = | E- IaRa=E- IL+IshRa volt | (4) |
| --- | --- | --- | --- |

Therefore, external characteristic curve will lie below the internal characteristic curve by an amount equal to drop in the armature circuit [i.e., IL+IshRa ] as shown in Fig. 4 (ii). Note. It may be seen from the external characteristic that the change in terminal voltage from no-load to full load is small. The terminal voltage can always be maintained constant by adjusting the field rheostat R automatically.

  - Critical Field Resistance for a Shunt Generator

Now, connect the field windings back to the armature and run the machine as a shunt generator. Due to residual magnetism in the poles, some e.m.f. and hence current, would be generated. This current while passing through the field coils will strengthen the magnetism of the poles (provided field coils are properly connected as regards polarity).

This will increase the pole flux, which will further increase the generated e.m.f. Increased e.m.f. means more current which further increases the flux and so on. This mutual reinforcement of e.m.f. and flux proceeds on till equilibrium is reached at some point like P (Fig. 5). The point lies on the resistance line OA of the field winding. Let R be the resistance of the field winding. Line OA is drawn such that its slope equals the field winding resistance i.e. every point on this curve is such that volt/ampere = R.

Fig. 5

The voltage OL corresponding to point P represents the maximum voltage to which the machine will build up with R as field resistance. OB represents smaller resistance, and the corresponding voltage OM is slightly greater than OL. If field resistance is increased, then slope of the resistance line increased, and hence the maximum voltage to which the generator will build up at a given speed decreases. If R is increased so much that the resistance line does not cut the O.C.C. at all (like OT ), then obviously the machine will fail to excite i.e. there will be no ‘build up’ of the voltage. If the resistance line just lies along the slope, then with that value of field resistance, the machine will just excite. The value of the resistance represented by the tangent to the curve, is known as critical resistance Rc  for a given speed..

  - Critical Speed (NC)

The critical speed of a shunt generator is the minimum speed below which it fails to excite. Clearly, it is the speed for which the given shunt field resistance represents the critical resistance. In Fig. 6, curve 2 corresponds to critical speed because the shunt field resistance (Rsh) line is tangential to it. If the generator runs at full speed N, the new O.C.C. moves upward and the R'sh line represents critical resistance for this speed.

Speed Critical resistance In order to find critical speed, take any convenient point C on excitation axis and erect a perpendicular so as to cut Rsh and R'sh lines at points B and A respectively. Then,

Fig. 6

| E2E1 | = | N2N1 | (5) |
| --- | --- | --- | --- |

| E2 | = | N2N1 E1 | (6) |
| --- | --- | --- | --- |

  - Conditions for Voltage Build-Up of a Shunt Generator

The necessary conditions for voltage build-up in a shunt generator are:

(i) There must be some residual magnetism in generator poles.

(ii) The connections of the field winding should be such that the field current strengthens the residual magnetism.

(iii) The resistance of the field circuit should be less than the critical resistance. In other words, the speed of the generator should be higher than the critical speed.

اعزائي الطلبة بعض التوضيحات بالعربي لفهم الحاضرة